# Safe Work Procedure – Excavator

Personal Protective Equipment:	Hearing protection while operating machine

Hi-Vis clothing when outside machine

Substantial appropriate footwear at all times

Gloves when doing maintenance or servicing

Procedures:

- Inspect machine to ensure it is in safe operating condition before using.

- Wear seat-belts while operating machine.

- Keep doors closed so that guarding is effective when working.

- Ensure good housekeeping is maintained (no loose articles in cab)

- Operate at a safe speed.

- Exercise due caution while working on hillsides.

  - Do not travel across a slope that is too steep for maintaining proper stability of the machine.

  - Confine travel to up and down slope.

  - When traveling across any slope, avoid running over logs, chunks, stumps, etc. which could cause the machine to become unstable.

  - Review and follow the safe work procedures for operating machinery on steep slopes.

  - If steep slope procedures unavailable do not operate machine on slopes greater than 30%.

- Ensure a radio man-check system is established.

- If you leave the machine notify your co-workers by radio.

- If at any time the machine becomes unstable, shut it down, and request assistance.

- Always enter and leave the machine in a safe manner. Use 3 point mount/dismount. Use the handholds for stability.

- Beware of the slipping hazards that exist, particularly in the winter especially when standing on the deck refueling.

- Always service your machine in the clear of dangerous trees.

- Follow the lock-out/tagout procedures while conducting maintenance work on the machine.(overleaf).

- Do not enter an active falling area, stay a minimum of two tree lengths away.

- Do not work in areas where there is a danger of pushing trees, rocks or other debris into an active work area.

- When pushing trees over, remove the tension out of the trees using the blade or winch so they may be bucked off without danger of tree springing back.

- Build skid trails wide enough for skidder operation.

- Slope trails towards the inside bank, never outwards.

| Excavator Lockout - Tagout (One person working on machine) Shut down procedure: - Notify other affected employees. - Lower bucket to ground. - Shut down engine. - Set hydraulic lockout lever. - Turn off master switch. - Put lock and tag on master switch. - Test to verify zero energy (electrical-hydraulic-gravity). Start-up procedure: - Remove lock from master switch. - Start machine. | Excavator Lockout - Tagout (If more than one person working in machine) Shut down procedure: - Notify other affected employees. - Lower bucket to ground. - Shut down engine. - Set hydraulic lockout lever. - Turn off master switch. - Each worker attach personal lock and tag to scissor lockout hasp on master switch. - Test to verify zero energy (electrical-hydraulic-gravity). Start-up procedure: - Each employee removes personal lock from scissor lockout hasp on master switch. - Start machine when all locks removed. | Excavator Tagout For machine without master switch Shut down procedure: - Notify other affected employees. - Apply parking brake. - Lower bucket to ground. - Shut down engine. - Key out and in pocket. - Put lockout tag initialed by all workers on ignition switch. - Test to verify zero energy (electrical-hydraulic-gravity). Start-up procedure: - Each employee crosses off their initials on lockout tag when their work is completed - Start machine when all initials on tag crossed off. |
| --- | --- | --- |
| Lockout tag (front) | Scissor lockout hasp – with marked locks | Lockout tag (back) |

ADDITIONAL SWP NOTES

- Source: BC Forest Safety Council