# Safe Work Procedure – Feller Buncher

Personal Protective Equipment:	Hearing protection while operating machine

Hi-Vis clothing when outside machine

Substantial appropriate footwear at all times

Gloves when doing maintenance or servicing

Procedures:

- Inspect machine to ensure it is in safe operating condition before using.

- Wear seat-belts while operating machine.

- Keep doors closed so that guarding is effective when working.

- Ensure good housekeeping is maintained (no loose articles in cab).

- Operate at a safe speed.

- Maintain at least two tree lengths from other work and equipment at all times while cutting.

- Exercise due caution while working on hillsides.

  - Do not travel across a slope that is too steep for maintaining proper stability of the machine.

  - Confine travel to up and down slope.

  - When traveling across any slope, avoid running over logs, chunks, stumps, etc. which could cause the machine to become unstable.

  - Review and follow the safe work procedures for operating machinery on steep slopes.

  - If steep slope procedures unavailable do not operate machine on slopes greater than 30%.

- Ensure the tracks are adequately caulked with ice lugs for winter operations.

- Ensure a radio man-check system is established.

- If you leave the machine notify your co-workers by radio.

- If at any time the machine becomes unstable, shut it down, and request assistance.

- Traffic control must be set up when falling trees within two tree lengths of roadways and other work area.  The roadway must be positively blocked with signs in place unless traffic control persons are employed.

- Do not try to fell trees which are larger than what the machine is designed for.

- Always enter and leave the machine in a safe manner.  Use 3 point mount/dismount. Use the handholds for stability.

- Beware of the slipping hazards that exist, particularly in the winter especially when standing on the deck refueling.

- Always service your machine in the clear of dangerous trees.

- Follow the lock-out/tagout procedures while conducting maintenance work on the machine.(overleaf).

| Feller Buncher Lockout - Tagout (If one person working on machine) Shut down procedure: - Notify other affected employees. - Power off head. - Wait for saw to stop or stop against stump. - Lower head to the ground. - Shut down engine. - Set hydraulic lockout lever. - Turn off master switch. - Put on personal lock and tag on master switch. - Test to verify zero energy (electrical, hydraulic, & gravity). Start-up procedure: - Remove personal lock from master switch. - Start machine. | Feller Buncher Lockout - Tagout (If two or more persons working on machine) Shut down procedure: - Notify other affected employees. - Power off head. - Wait for saw to stop or stop against stump. - Lower head to the ground. - Shut down engine. - Set hydraulic lockout lever. - Turn off master switch. - Each worker attaches personal lock to scissor lockout hasp on master switch. - Test to verify zero energy (electrical, hydraulic, & gravity). Start-up procedure: - Each employee removes own lock from scissor lockout hasp on master switch - Start machine when all locks removed. | Feller Buncher Tagout For buncher without master switch Shut down procedure: - Notify other affected employees. - Power off head. - Wait for saw to stop or stop against stump. - Lower head to ground. - Shut down engine. - Key out and in pocket. - Put lockout tag initialed by all workers on ignition switch. - Test to verify zero energy (electrical, hydraulic, & gravity). Start-up procedure: - Each employee crosses off their initials on lockout tag when their work is completed. - Start machine when all initials on tag crossed off. |
| --- | --- | --- |
| Lockout tag (front) | Scissor lockout hasp – with marked locks | Lockout tag (back) |

ADDITIONAL SWP NOTES

- Source: BC Forest Safety Council