# Safe Work Procedure – Grapple Skidder

Personal Protective Equipment:	Hearing protection while operating machine

Hi-Vis clothing when outside machine

Substantial appropriate footwear at all times

Gloves when servicing

Procedures:

- Check to ensure machine is in safe operating condition before using.

- Wear seat-belts when operating machine.

- Ensure good housekeeping to prevent slipping or tripping when entering or leaving machine.

- All fire extinguishers and other items in cab must be secured in a safe location. Do not carry loose articles in the cab.

- Do not enter an active falling area. Stay a minimum of two tree lengths away. Do not skid trees past a faller or other active falling areas.

- Travel at safe speed with or without a drag.

- Try to align machine straight with bundles when picking up a drag.

- Use caution when making turns when skidding logs

- When entering the landing, make sure workers are in the clear and you are given approval to enter.

- Lower the blade and set the parking brake before leaving the machine.

- Follow the lock-out procedures while conducting maintenance work on the machine.

- Always enter and leave the machine in a safe manner. Use the handholds for stability and beware of slipping hazards that exist, particularly in winter.

SKIDDING ON SLOPES:

- Do not attempt to travel across a slope that is too steep for maintaining proper balance of the machine.

- Confine your travel to straight up and down slopes when steepness is a problem. Any slope greater than 35% shall not be traveled without specific steep slope safe work procedures in place. These procedures must be reviewed before operating on steep slopes.

- Avoid running over chunks and stumps because of increased potential for machine upset.

- Use caution when traveling on trails by maintaining a safe distance from the outer edge of the trail.

- Use tire chains for traction on steep ground and when slippery.

- Be aware of limbs and chunks that may catch in the chains.

- When traveling down steep slopes, make sure the blade does not hook on a stump or rock, causing the machine to swing sideways and upset.

- If you encounter unsafe skidding conditions inform your supervisor and alternate methods will be initiated.

- If at any time the machine is unstable, shut it down and request assistance.

| Grapple Skidder Lockout -Tagout (If one person working on machine) Shut down procedure: - Notify other affected employees. - Apply parking brake. - Lower grapple and blade to ground. - Shut down engine. - Turn off master switch. - Put personal lock and tag on master switch. - Test to verify zero energy (electrical-hydraulic-gravity). Start-up procedure: - Remove personal lock from master switch. - Start machine. | Grapple Skidder Lockout -Tagout (If more than one person working on machine) Shut down procedure: - Notify other affected employees. - Apply parking brake. - Lower grapple and blade to ground. - Shut down engine. - Turn off master switch. - Each worker attach personal lock and tag to scissor lockout hasp on master switch. - Test to verify zero energy (electrical-hydraulic-gravity). Start-up procedure: - Each employee removes own lock from scissor lockout hasp on master switch. - Start machine when all locks removed. | Grapple Skidder Tagout For skidder without master switch Shut down procedure: - Notify other affected employees. - Apply parking brake. - Lower grapple and blade to ground. - Shut down engine. - Key out and in pocket. - Put lockout tag initialed by all workers on ignition switch. - Test to verify zero energy (electrical-hydraulic-gravity). Start-up procedure: - Each employee crosses off their initials on lockout tag when their work is completed. - Start machine when all initials on tag crossed off. |
| --- | --- | --- |
| Lockout tag (front) | Scissor lockout hasp – with marked locks | Lockout tag (back) |

ADDITIONAL SWP NOTES

- Source: BC Forest Safety Council